ANSYS SIMULATION TUTORIAL

For use simulating Acoustic Wavenumber Spectroscopy Models

Niall O’Dowd 2016

| 1) | - Open ANSYS Workbench - Image shows probable location of ANSYS Workbench - May be named RunWB2.exe |  |
| --- | --- | --- |
| 2) | - Drag a Harmonic Response Analysis System into the project schematic |  |
| 3) | - Double click Engineering Data |  |
| 4) | - Select Engineering Data Sources - Select General Materials - In the Outline of General Materials window, we can select our material and fine tune different properties - Once we have found our desired material, click the plus symbol in the Add column - When we are done adding our material, click back to the Project tab in the top left corner | 2 1 |
| 5) | Back inside the project tab… - It’s time to make our model. Basic ANSYS geometry tutorials can be found online. - Double-click the Geometry tab in the harmonic response module. THINGS TO REMEMBER - Scan surface must be located parallel to the xyPlane. It’s best to select this plane to begin your plate sketching - We must include a raised surface to apply the source excitation |  |
| 6) | - Here is an example of a basic plate - Notice the orientation of the surface. It is parallel to the xyPlane, but offset in the Z direction. This will be the surface that we will take our vibration data from - Also included is a raised cylinder for the excitation source - Once done modelling the plate, close DesignModeler | Plate Origin Scan surface Transducer |
| 7) | - The rest of the simulation process will be done after double clicking any of the Setup, Solution, and Results options |  |
| 8) | - Inside the Mechanical window, select Geometry, select Solid, and choose our material added earlier in the tutorial - The question mark next to Geometry should become a green checkmark |  |
| 9) | - Next, we will select the parameters for our mesh and preview it - Under sizing, highlight the Element Size value - Once a value is input into element size, right click Mesh in the outline window, and select update - The number of nodes is listed in Statistics in mesh details. This is useful when trying to gage how long simulations will take. - Once the mesh is generated, there should be a green checkmark next to mesh |  |
| 10) | - Now we will select the harmonic response parameters by selecting Analysis Settings in the outline window - If we want only the response of a plate with a single excitation, set Solution Intervals to 1 - The Range Minimum can be set to anything below our desired excitation, which will be our range maximum - Solution Method must be changed to Full - Select Damping Controls and change Constant Damping Ratio to a desired value (usually 0.001-0.01) - There is still another step in establishing the Harmonic Response, so the question mark remains |  |
| 11) | - Now we will establish the pressure which drives the excitation. After right clicking Harmonic Response InsertPressure click on our raised cylinder and click apply for the Geometry option - Select a value for Magnitude (Usually around .1 mPa) - There should be a yellow lightning bolt next to Harmonic Response |  |
| 12) | - Now we will create a surface solution. Right click SolutionInsertDeformation Directional - Select Z axis for the Orientation option |  |
| 13) | - Now we will set up the export files. Right click SolutionInsertCommands. Repeat until there are 3 Commands (APDL) objects under solution - Copy and paste the scripts (starting with ‘/post’) at the end of this document into the corresponding command pages - There should be only yellow lightning bolts and green checkmarks in our outline |  |
| 14) | - Click Solve located in the top menu |  |
| 15) | - The output files are located inside the accompanying folder for your simulation, named YourFile_filesdp0SYS MECH - These files can be uploaded into MATLAB for image processing |  |

!Commands (ADPL)

/post1

set,,,1,1			! Set the set number and Imaginary part

! Last Digit (0 = Real part, 1 = Imaginary Part)

*get,NNUMMAX,node,,num,max		! Get max node number

*del,NMASK			! Delete NMASK array if it exits

*del,NARRAY			! Delete NARRAY array if ti exits

*dim,NMASK,array,NNUMMAX		! Define NMASK array

*dim,NARRAY,array,NNUMMAX,5		! Define NARRAY array to hold results

*vget,NMASK(1),node,all,nsel,-1		! Get status of selected nodes

! 1 = selected, 0 = undefined, -1 = unselected

*vmask,NMASK(1)		! Use NMASK as making array

! If NMASK(i) > 0.0, perform operation "1"

! If NMASK(i) < 0.0, do not perform on "i"

*vget,NARRAY(1,1),node,all,loc,x		! Get LX for nodes & Fill first column

*vget,NARRAY(1,2),node,all,loc,y		! Get LY for nodes & Fill seound column

*vget,NARRAY(1,3),node,all,loc,z		! Get LZ for nodes & Fill third column

*vget,NARRAY(1,4),node,all,u,z		! Get UZ for nodes & Fill forth column

*vfil,NARRAY(1,5),ramp,all,1		! Fill vector from 1 to NNUMMAX (node no.)

*cfopen,imaginary,txt	! Create text file (Path is same with rst file)

*vwrite			! Write headers (‘x’ tells how many spaces to put between headers)

('Node_no.',2x,'x-loc',2x,'y-loc',2x,'z-loc',2x,'z-disp')

*vwrite,NARRAY(1,5),NARRAY(1,1),NARRAY(1,2),NARRAY(1,3),NARRAY(1,4)	! Write the info from your array to the text file

(F10.0,2x,E15.8,2x,E15.8,2x,E15.8,2x,E13.5)			! Format the output

finish

*cfclose,imaginary,txt		! Close the opened file

!Commands (ADPL) 2

/post1

set,,,1,0			! Set the set number and real part

! Last Digit (0 = Real part, 1 = Imaginary Part)

*get,NNUMMAX,node,,num,max		! Get max node number

*del,NMASK			! Delete NMASK array if it exits

*del,NARRAY			! Delete NARRAY array if ti exits

*dim,NMASK,array,NNUMMAX		! Define NMASK array

*dim,NARRAY,array,NNUMMAX,5		! Define NARRAY array to hold results

*vget,NMASK(1),node,all,nsel,-1		! Get status of selected nodes

! 1 = selected, 0 = undefined, -1 = unselected

*vmask,NMASK(1)		! Use NMASK as making array

! If NMASK(i) > 0.0, perform operation "1"

! If NMASK(i) < 0.0, do not perform on "i"

*vget,NARRAY(1,1),node,all,loc,x		! Get LX for nodes & Fill first column

*vget,NARRAY(1,2),node,all,loc,y		! Get LY for nodes & Fill seound column

*vget,NARRAY(1,3),node,all,loc,z		! Get LZ for nodes & Fill third column

*vget,NARRAY(1,4),node,all,u,z		! Get UZ for nodes & Fill forth column

*vfil,NARRAY(1,5),ramp,all,1		! Fill vector from 1 to NNUMMAX (node no.)

*cfopen,real,txt	! Create text file (Path is same with rst file)

*vwrite			! Write headers (#x tells how many spaces to put between headers)

('Node_no.',2x,'x-loc',2x,'y-loc',2x,'z-loc',2x,'z-disp')

*vwrite,NARRAY(1,5),NARRAY(1,1),NARRAY(1,2),NARRAY(1,3),NARRAY(1,4)	! Write the info from your array to the text file

(F10.0,2x,E15.8,2x,E15.8,2x,E15.8,2x,E13.5)			! Format the output

finish

*cfclose,real,txt		! Close the opened file

!Commands (ADPL) 3

/post1

set,,,1,1			! Set the set number and Imaginary part

*get,NNUMMAX,elem,,num,max		! Get max element number

*del,NMASK			! Delete NMASK array if it exits

*del,NARRAY			! Delete NARRAY array if it exits

*dim,NMASK,array,NNUMMAX		! Define NMASK array

*dim,NARRAY,array,NNUMMAX,11		! Define NARRAY array to hold results

*vget,NMASK(1),elem,all,esel,-1		! Get status of selected elements

! 1 = selected, 0 = undefined, -1 = unselected

*vmask,NMASK(1)			! Use NMASK as making array

! If NMASK(i) > 0.0, perform operation "1"

! If NMASK(i) < 0.0, do not perform on "i"

*vget,NARRAY(1,1),elem,all,node,1		! Get node no for nodes & Fill first column

*vget,NARRAY(1,2),elem,all,node,2		! Get node no for nodes & Fill second column

*vget,NARRAY(1,3),elem,all,node,3		! Get node no for nodes & Fill third column

*vget,NARRAY(1,4),elem,all,node,4		! Get node no for nodes & Fill forth column

*vget,NARRAY(1,5),elem,all,node,5		! Get node no for nodes & Fill fifth column

*vget,NARRAY(1,6),elem,all,node,6		! Get node no for nodes & Fill sixth column

*vget,NARRAY(1,7),elem,all,node,7		! Get node no for nodes & Fill seventh column

*vget,NARRAY(1,8),elem,all,node,8		! Get node no for nodes & Fill eighth column

*vget,NARRAY(1,9),elem,all,node,9		! Get node no for nodes & Fill ninth column

*vget,NARRAY(1,10),elem,all,node,10		! Get node no for nodes & Fill tenth column

*vfil,NARRAY(1,11),ramp,all,1		! Fill vector from 1 to NNUMMAX (element no.)

*cfopen,3_Elements_Information,txt		! Create text file (Path is same with rst file)

*vwrite			! Write headers (#x tells how many spaces to put between headers)

('Element_no.',2x,'node1',2x,'node2',2x,'node3',2x,'node4',2x,'node5', 2x,'node6',2x,'node7',2x,'node8',2x,'node9',2x,'node10')

*vwrite,NARRAY(1,11),NARRAY(1,1),NARRAY(1,2),NARRAY(1,3),NARRAY(1,4),NARRAY(1,5), NARRAY(1,6),NARRAY(1,7),NARRAY(1,8),NARRAY(1,9),NARRAY(1,10)		! Write the info from your array to the text file

(F10.0,2x,F8.0,2x,F8.0,2x,F8.0,2x,F8.0,2x,F8.0,2x,F8.0,2x,F8.0,2x,F8.0,2x,F8.0,2x,F8.0)	! Format the output

finish

*cfclose,3_Elements_Information,txt